J.M.Ryder and Associates, Terrain Analysis Inc.

P.O.Box 45005, Dunbar P.O., Vancouver, B.C.  Canada,  V6S 2M8        Ph 604-736-4211  Fax 604-736-5101

Memo

To: Claire, Jen, Kristie

From: June

Date:  Sept. 13. 1996

Re: Mid-Coast Mapping Project.

When you have time between field projects, go ahead with mapping of fluvial hazards (Claire), landslides (Kristie) and snow avalanches (Jen). (We will have to put a big effort into this project as the field season ends, to catch up and then get it finished by the end of the year.)

The most urgent task is to check how the mapping that we have started to do will appear on the 1:100 000 scale “working map”, and then reduce this to the final 1:250 000. I have received a 1:100 000 scale map for the southeast part of 92 M.

So, today or Monday, could you please taken a couple of hours (only) and transfer some of “your” polygons/symbols (see below) onto this map. Keep colours distinct (the map is highly coloured -- the next version will be more subdued): e.g., use black for landslides, purple for avalanches, and green for fluvial.

Outcome of discussions I had with Denny Maynard (co-director of this project):

Avalanches: Jen:

Use polygons for areas where avalanche tracks are >50% by area of the slopes. Elsewhere, use arrows for “medium” (2-5 mm wide) and “large” (>5 mm wide) tracks. This is very similar to your prelim. mapping on photo 66, but don’t separate runout zones in polys or on arrows, (after all, the entire avalanche area is hazardous). See how this fits on the map.

Landslides: Kristie:

Minimum size of polygon on air photos is 5 cm2 , with minimum width of 1 cm. For now, label polys as you did on photo 215, but see (c) below.

(1) Delimit polygons that include initiation zones, i.e., our usual U polys.

(2) Delimit polygons for initiation zones and runout zones.

(3) Delimit polygons for runout zones.

For (1) and (2):

(a) write number of initiation points beside or in the polygon (this is so we can use GIS to determine slide “density”);

(b) mark initiation sites with “x” (tiny crosses);

(c) if failures are occurring in thick drift, mark poly with “D”;

(d) distinguish unstable scarps (e.g., terrace scarps, other scarps formed by river undercutting) by using scarp symbol.

Fluvial: Claire:

OK as before.