Objectives:

Map with expanded legend to show hazards and constraints; report - analysis

map to be used for broad forest zoning (enhanced vs general forestry)

regional land use planning

1:250 000 presentation scale

slope hazards, debris flows, slides, etc. , both initiation zones and runout zones

snow avalanches

fluvial hazards, hydrologic,

extraordinary floods and flows (ice dams, moraine dams)

sensitive soils

Pilot: develop methods, test on sample area

do summary report on methods

Tasks

(1) Review and check existing data in GIS:

TRIM maps, bedrock geology, bioetc maps

check if bioetc. related to snow depth

check scale of bedrock geology; group lithologies to simplify

experimental slope maps

check capabilities re slope/bedrock combinations, etc.		(Victoria visit)

(2) Compile and review: existing non-GIS information: terrain maps, surf.geol., terrain stability maps, engineering studies, relevant literature, air photos

(3) Develop legend and methodology for air photo interpretation, transfer to TRIM

compare reliability of hand transfer and photo transfer

"           "     of api with any existing stability mapping

"         '"   of api with any more recent large-scale photos to assess mapping reliability

(4) Compile and develop interpretations and presentations leading to hazard map

GIS experiments							(Victoria visit)

(5) Report: recommending standard methodology for maps and report(e.g., analysis of regional patterns)