DESCRIPTION OF KNOWN FISH DISTRIBUTION MAP DATA

June 21, 2001

Updated: October 3, 2001

D.F.Philip  email don.philip@gems4.gov.bc.ca      Telephone: (250) 387-9558

Each of the 246 watershed groups in BC has three categories of "Known" fish distribution data for streams and a Fish lakes table associated with it.

The "known" fish distribution data for streams stored at this site are in the form of ARC/INFO point coverages and arc/line coverages that have been converted to ARC/INFO interchange files (*.e00) and then zipped.The three coverages include a arc/line coverage and two point coverages.These coverages can be added as themes in ARCVIEW 3.x or ARCEXPLORER. The line coverage contains individual

line segments (arcs) which represent fish distribution zones or spawning zones with associated attribute data that can be selected and displayed. One of the point coverages represents site locations on streams where fish were caught or observed and the other point coverage uses a point symbol located  at the mouth

of a stream to indicate a fish species is known to occurr somewhere in the stream but specific locations are not available.The Fish lakes data table is provided in two formats: INFO and dbase; and is located at the web site:

ftp://ftpfsh.env.gov.bc.ca/pub/outgoing/fiss_events/fish_dist/lakes/

On October 3, 2001 a SHAPE subdirectory was added to the EVENTARCS directory. The SHAPE directory contains shape files created from the three kinds of coverages describe above for each watershed group. The shape files allow a user to easily display and query the Known fish distribution data using ARCVIEW or ARCEXPLORER.

The Known Fish Distribution file naming convention uses the Watershed Group "wsdg" prefix where "wsdg" is replaced by a valid 4-letter watershed group code. A complete list of the 246 valid watershed group codes in BC is available at the web site: http://www.bcfisheries.gov.bc.ca/fishinv/basemaps-watershed.html

These files are organized by BC Ministry of Environment Regions (1 - 8) and are located under directories named for the eight regions r1 to r8.

Zipped file    Unzipped file    Description

Name              Name

"wsdg"evz.zip  "wsdg"evz.e00  Interchange file of an ARC/INFO arc COVERAGE

of fish spawning/distribution zones

"wsdg"evp.zip  "wsdg"evp.e00  Interchange file of an ARC/INFO point COVERAGE

of fish instream sample site locations

"wsdg"evs.zip  "wsdg"evs.e00  Interchange file of an ARC/INFO point COVERAGE

with a symbol marker at the stream mouth 							indicating the stream contains

the fish species described.

"wsdg"l.zip    "wsdg"l.e00    Interchange file of an INFO TABLE containing fish 					data for lakes -This zip file is located in a 						separate directory at:                     ftp://ftpfsh.env.gov.bc.ca/pub/outgoing/fiss_events/fish_dist/lakes/

"wsdg"ldb.zip  "wsdg"ldb.dbf  dBASE III compatable file containing fish data for 					lakes (the attribute data is identical to that in 					the INFO table "wsdg"l above) - This zip file is

located in a separate directory at:                             ftp://ftpfsh.env.gov.bc.ca/pub/outgoing/fiss_events/fish_dist/lakes/

* Shape files  ( for use with ARCVIEW or ARCEXPLORER)

"wsdg"evz.shp                	The Shape files for fish spawning/distribution

"wsdg"evz.shx			zones.

"wsdg"evz.dbf

"wsdg"evp.shp                	The Shape files for fish instream sample site

"wsdg"evp.shx			locations.

"wsdg"evp.dbf

"wsdg"evs.shp                	The Shape files for stream mouth markers

"wsdg"evs.shx                	indicating the stream contains the fish

"wsdg"evs.dbf              	species described.

For example OKAN - the Okanagan River Watershed Group is found in MELP Region 8 so the appropriate Known fish distribution zip files required to display known fish distributions for the Okanagan River Watershed Group are: okanevz.zip, okanevp.zip and okanevs.zip AND the lakes data file okanl.zip (or okanldb.zip) located under the Lakes directory at:

ftp://ftpfsh.env.gov.bc.ca/pub/outgoing/fiss_events/fish_dist/lakes/

The equivalent shape files to download for the streams would be okanevz.shp, okanevz.shx, okanevz.dbf,okanevp.shp, okanevp.shx, okanevp.dbf, okanevs.shp, okanevs.shx and okanevs.dbf.

DOWNLOADING AND DATA RECOVERY INSTRUCTIONS

To obtain the fish distribution coverages for a watershed group:

1. Locate the correct MELP region containing the area of interest and the 4-letter watershed group code. Next select the correct MELP Region directory/folder r1 to r8. From within the desired Region folder select the zipped Fish Distribution files for the Watershed group of interest.

2. Download the selected "wsdg"evz.zip, "wsdg"evp.zip, "wsdg"evs.zip and "wsdg"l.zip (or "wsdg"ldb.zip) files to a local directory. Note: To obtain the lakes fish data download the "wsdg"l.zip or the "wsdg"ldb.zip file located under a separate lakes directory at:

ftp://ftpfsh.env.gov.bc.ca/pub/outgoing/fiss_events/fish_dist/lakes/

3. Use pkzip or winzip to unzip the zipped files to recover the "wsdg"evz.e00,"wsdg"evp.e00,"wsdg"evs.e00,and "wsdg"l.e00 files.

4. Use the ARC/INFO import command to recover the "wsdg"evz, "wsdg"evp, "wsdg"evs coverages and the "wsdg"l INFO table for use in workstation ARC/INFO OR use the ARCVIEW Import71 utility to restore the coverages and lakes INFO table for use in ARCVIEW.

* To obtain the fish distribution SHAPE files for a watershed group:

1. From the SHAPE directory located under the eventarcs directory locate the correct MELP region directory/folder r1 to r8. From within the desired Region folder select the set of nine shape files of fish distributions in streams

for the watershed group of interest.Download the nine shape files to the SAME local directory. Example: To get all the known fish distribution data for streams for the okan watershed group download from directory EVENTARCS/SHAPE/R8 the nine files: Okanevz.shp, Okanevz.shx, Okanevz.dbf, Okanevp.shp, Okanevp.shx, Okanevp.dbf, Okanevs.shp, Okanevs.shp and Okanevs.dbf

/* ***

The Known Fish Distribution Spatial Data are georeferenced to the BC 1:50K Digital Watershed Atlas.

The Map Projection and Coordinate System Description is:

Projection   ALBERS

Datum        NAD83

Units        METERS       Spheroid     GRS1980

Parameters:

1st standard parallel                 50  0  0.000

2nd standard parallel                 58  30 0.000

central meridian                      -126 0  0.000

latitude of projection's origin        45  0  0.000

false easting (meters)              1000000.00000

false northing (meters)                   0.00000

/* ***

DESCRIPTION OF KNOWN FISH DISTRIBUTION ATTRIBUTES

Item/Field Name

---------------

COVERAGE_ID - The four letter code of the BC Watershed Atlas Group in which the fish species occurs.

LOCATION_TYPE - The category of fish data mapped.

Categories are:

Z = Fish distibution or spawning zone

P = Point of Fish Observation or Fish Sample Site

S = Fish species known to occur somewhere in the stream

GIS_BL# - The BC Watershed Atlas ROUTE Number for the stream (ie. blueline) where the fish species occurs.

GIS_UPSTREAM_MEA - For Location_type = 'Z' this value is the distance in metres measured upstream from the stream's mouth to the UPSTREAM BOUNDARY of the fish distribution or spawning zone. For Location_type = 'P' this value is the distance in metres measured upstream from the stream's mouth to the point location representing a fish observation or fish sample site. For Location_type = 'S' this value is always zero to locate a point symbol at the stream mouth that represents the presence of this fish species somewhere in the stream.

GIS_DOWNSTREAM_M - Applies ONLY where Location_type = 'Z' fish data where the non-zero value refers to the distance in metres measured upstream from the streams's mouth to the DOWNSTREAM BOUNDARY of the fish distribution or spawning zone.

SPECIES_CODE - Fish Species Code. Refer to the publication entitled "Fisheries Information Summary System (FISS): Data Compilation and Mapping Procedures" for a complete list of the fish species codes for B.C. This publiaction is located at:

http://www.for.gov.bc.ca/ric/PUBS/Aquatic/fiss/index.htm

STOCK_CHAR - The characteristic residence and movement patterns of the fish stock.

Codes are:

AD = Adfluvial  - Fish species spends part of life in lake and part in river

AN = Anadromous - Fish species spawns in river, then migrates to the ocean

FL = Fluvial    - Fish species is resident in river for entire life

RS = Resident   - Fish species is resident in lake for entire life

NS = Not specified - Fish species unknown

STOCK - Distinguishes between fish of the same species which occupy more than one waterbody. (eg. Nanoose cutthroat)

STOCK_TYPE - Distinguishes between fish of the same stock based on timing of migrations.(e.g. odd/even pinks, summer/winter steelhead, early/late chinook)

MGMT_CLASS_CODE - Stock Management Classification. Identifies the nature of the stock and the degree of human intervention.

ACTIVITY_CODE - Describes the presence of fish and the major activity of those fish.

Codes are:

HOL = Holding or Staging Area

NFC = No Fish Caught

NFO = No Fish Observed (YUKON ONLY)

NFP = No Fish Present

OBL = Fish observed at this Point or Zone

REA = Rearing Location

SPE = Spawning in Estuary

SPL = Spawning Location

SPM = Major Spawning Location

UNC = Unconfirmed Siting (YUKON ONLY)

* GIS = This value is used for cartographic display purposes to identify

the zone on a stream that represents the upper limit of the known

distribution of an anadromous fish species from the ocean.(eg. used to     	   display the upper known limit of Coho distribution on the Nanaimo 	 	   River)

COMMENTS - Comments regarding details of distribution such as a unique spawning location or heavy spawning.

REF_NO1 - FISS data source reference number

REF_NO1_DATE - FISS data source reference number 1 date

REF_NO2 - A second FISS data source reference number

REF_NO2_DATE - FISS data source reference number 2 date

THEME_NAME - The name of the map theme category to which the fish species is assigned.

Categories are:

Salmon      = includes the 5 species of Pacific Salmon

Sport Fish  = includes all species in BC considered as Sport or Gamefish

Other       = includes all other species of fish in BC that are not Salmon 		     and not Sport Fish

THEME_ATTRIBUTE - The BC Ministry of Environment line or marker symbol number used to map the fish species category and location_type assigned to the THEME_NAME and LOCATION_TYPE fields.

Location Type   Symbol Number  	Symbol Description          Theme Name

Z                  34               Red Line         	     	Salmon

Z                  36               Blue Line             	Sport Fish

Z                  35               Green Line            	Other Fish

P                  576              Solid Red Triangle    	Salmon

P                  575              Solid Blue Triangle   	Sport Fish

P                  574              Solid Green Triangle  	Other Fish

S                  572              Hollow Red Triangle   	Salmon

S                  571              Hollow Blue Triangle  	Sport Fish

S                  570              Hollow Green Traingle 	Other Fish

OFFSET2 - Applies ONLY where location_type = 'Z'. The offset distance of a fish zone from the stream centreline. This offset value is used in ARCVIEW 3.x for cartographic display purposes to distinguish the zone from overlapping ZONES of the other two fish species categories (for example on a Known fish distribution map a Salmon spawning zone can be distinguished from an overlapping Sport Fish or Other Fish zone).

OFFSET1 - Applies ONLY where location_type = 'Z'. The offset distance of a fish zone from the stream centreline. This offset value is used in Unix based WORKSTATION ARC/INFO for cartographic display purposes to distinguish the zone from overlapping ZONES of the other two fish species categories (for example on a Known fish distribution map a Salmon spawning zone can be  distinguished from an overlapping Sport Fish or Other Fish zone).

BL_KEY - The BC Watershed Atlas number that uniquely identifies a stream channel in a Watershed Group. Note that a main channel with secondary channels such as side channels and  distributaries will each have unique bl_keys within the watershed group but the ws_code will be the same for each of these channels.

MAPNID_A - The unique identfier for the POINT location of a fish map feature OR for the UPSTREAM BOUNDARY location of a fish distribution or spawning zone.

MAPNID_B - The unique identifier for the DOWNSTREAM BOUNDARY location of a fish distribution or spawning zone.