DESCRIPTION OF FISHERIES MACRO-REACH MAP DATA

Updated: October 9, 2001

/* D.F.Philip  email don.philip@gems4.gov.bc.ca      Telephone: (250) 387-9558

Each of the 246 BC Watershed Atlas groups in BC has a set of macro-reach data asociated with it. The macro-reach data stored at this site are in the form of ARC/INFO arc/line coverages that have been converted to ARC/INFO interchange files (*.e00) and then zipped. These Macro-reach line coverages can be added directly as themes in ARCVIEW 3.x or ARCEXPLORER where individual line segments (arcs) that represent macro-reaches with associated attribute data can be selected and displayed.

The Macro-reach file naming convention uses the Watershed Group "wsdg" prefix where "wsdg" is replaced by a valid 4-letter watershed group code. To obtain a complete list of the 246 valid watershed group codes for BC go to the web site:

http://www.bcfisheries.gov.bc.ca/fishinv/basemaps-watershed.html

These files are organized by BC Ministry of Environment Regions (1 - 8).

Zipped file       Unzipped file   	Description

Name              Name

"wsdg"evm.zip     "wsdg"evm.e00   	Interchange file of an ARC/INFO COVERAGE of 						Macro-reaches as arcs

* Shape files ( for use with ARCVIEW or ARCEXPLORER)

"wsdg"evm.shp     The shape files for macro-reaches

"wsdg"evm.shx

"wsdg"evm.dbf

For example OKAN - the Okanagan River Watershed Group is found in MELP Region 8 so the macro-reach zip file called okanevm.zip is located in the folder/directory called r8/ .

* The equivalent shape files to download would be okanevm.shp, okanevm.shx and okanevm.dbf.

DOWNLOADING AND DATA RECOVERY INSTRUCTIONS

To obtain a Macro-reach line coverage for a watershed group:

1. Locate the correct MELP region containing the area of interest and the 4-letter watershed group code. Next select the correct MELP Region directory/folder r1 to r8. From within the desired Region folder select the desired Macro-reach Watershed group zipped file.

2. Download the selected "wsdg"evm.zip file to a local directory

3. Use pkzip or winzip to unzip a "wsdg"evm.zip file to recover the "wsdg"evm.e00 file

4. Use the ARC/INFO import command to restore the macro-reach coverage "wsdg"evm for use in workstation ARC/INFO OR use the ARCVIEW Import71 utility to restore this coverage for use in ARCVIEW.

* To obtain the macro-reach SHAPE files for a watershed group:

1. From the SHAPE directory located under the eventarcs directory locate the correct MELP region directory/folder r1 to r8.

2. From within the desired region folder select the set of three shape files of macro-reaches for the watershed group of interest.

3. Download the three shape files to the SAME local directory. Example: To get the macro-reach data for the OKAN watershed group download from the directory EVENTARCS/SHAPE/R8 the three files: Okanevm.shp, Okanevm.shx and Okanevm.dbf.

/* ***

The Macro-reach spatial data are georeferenced to the BC 1:50K Digital Watershed Atlas.

The Map Projection and Coordinate System Description is:

Projection   ALBERS

Datum        NAD83

Units        METERS       Spheroid     GRS1980

Parameters:

1st standard parallel                 50  0  0.000

2nd standard parallel                 58  30 0.000

central meridian                      -126 0  0.000

latitude of projection's origin        45  0  0.000

false easting (meters)              1000000.00000

false northing (meters)                   0.00000

/* ***

DESCRIPTION OF MACRO-REACH ATTRIBUTES

Item/Field Name

-----------

COVERGE_ID - The four letter code of the BC Watershed Atlas Group in which the macro-reach occurs

ROUTE_BL - The BC Watershed Atlas ROUTE Number for the stream (ie. blueline) where the macro-reach occurs

UPST_MEAS - The distance in metres measured upstream from the stream's mouth to the upstream boundary of the macro-reach

DWNST_MEAS - The distance in metres measured upstream from the stream's mouth to the downstream boundary of the macro-reach

MAP_SCALE - The scale of the NTS 1:50,000 Map used to delineate and describe the macro-reach

CHAN_TYPE1 - The PRIMARY channel type describing the macro-reach. This channel type is determined and/or inferred from 1:50K NTS maps.

Channel types:

AA = Alluvial, anastomosed; get islands; 1% or less slope; towards mouth

AB = Alluvial, braided; alluvial fan; 1-2% slope; towards head; gravel

AI = Alluvial, irregular; flat slope after steep bedrock (r).

AR = Alluvial, regular or tortuous meandering; almost always less than 1% 		  slope

L = Lake

R = Rock controlled; over 20% slope; steep.

* UG = Underground: Interpreted underground  stream segment  >500 m in length

* NM = Not Mapped: Interpreted stream segment > 500m in length is not visible 	  on the 1:50K NTS mapsheet or underground flow not certain;

* GL = Glacier; Interpreted stream segment > 500m in length is not visible 	  	  through a glacier.

* WU = Wetland, Unchanneled; Interpreted  stream segment through a wetland  > 	  500m in length

* HD = Human-made ditch defined as a macro-reach

* HF = Human-made flume defined as a macro-reach

* HC = Human-made canal defined as a macro-reach

Note : * = Is assigned to an interpreted stream segment not a true macro-reach. This code was introduced after the original province-wide macro-reach delineation.This code is required to distinguish a construction line segment from a true macro-reach. If CHAN_TYPE1 = UG, NM, GL, WU, HD, HF, or HC then values are not entered for CHAN_TYPE2 and CHAN_TYPE3.

CHAN_TYPE2 - The SECONDARY Channel Type describing the macro-reach.

Channel Types : Same as for CHAN_TYPE1 above with the exclusion of GL and WU. 			 These have been replaced by GI and WI.

* GI = Glacier; Interpreted stream segment < 500m in length is not visible 		  through a glacier

* WI = Wetland, Unchanneled; Interpreted stream segment through a wetland < 		  500m in length.

* UG = Underground: Interpreted underground stream segment <500 m in length

* NM = Not Mapped: Interpreted stream segment < 500m in length is not visible 	  on the 1:50K NTS mapsheet or underground flow not certain;

Note: * Used only when CHAN_TYPE1 = AA, AB, AI, AR, L or R

CHAN_TYPE3 - The TERTIARY Channel Type describing the macro-reach.

Channel Types : Same as for CHAN_TYPE1 above with the exclusion of GL and WU. 			 These have been replaced by GI and WI.

* GI = Glacier; Interpreted stream segment < 500m in length is not visible    	  through a glacier

* WI = Wetland, Unchanneled; Interpreted  stream segment through a wetland < 	  500m in length.

* UG = Underground: Interpreted underground  stream segment  <500 m in length

* NM = Not Mapped: Interpreted stream segment < 500m in length is not visible 	  on the 1:50K NTS mapsheet or underground flow not certain;

Note: * Only used when CHAN_TYPE1 and CHAN_TYPE2 = AA, AB , AI, AR, L or R.

VAL_FLAT_W - The Valley Flat Width : an area adjacent to the macro-reach with a slope of < = 7% and the slope of the stream  is <= 3%.  Measured on the 1:50K NTS Map in millimetres. Multiply this value by 50 to get ground units in metres.

ST_VAL_LEN -Stream Valley length : the valley length of the macro-reach

obtained by measuring up the axis of the valley from the lower break to the upper break of the macro-reach. Measured on the 1:50K NTS Map in millimetres. Multiply this value by 50 to get ground units in metres.

UPST_ELV - Elevation in metres of the upstream boundary of the macro-reach.

UPST_ELVI - Method used to interpret the upstream boundary elevation value.

Codes are:

C = Elevation of the upper macro-reach boundary was determined by NTS 1:50K 	Map contour interpolation

R = Elevation of the upper macro-reach boundary was interpolated based on 	calculation of the slope(ie. rise over run) using the KNOWN elevations of 	and the distance between the two macro-reach boundaries that exist 	immediately upstream and downstream of this upper boundary.

DWNST_ELV - Elevation in metres of the downstream boundary of the macro-reach.

DWNST_ELVI - Method used to interpret the downstream boundary elevation value

Codes are:

C =  Elevation of the lower macro-reach boundary was determined by NTS 1:50K 	 Map contour interpolation

R =  Elevation of the lower macro-reach boundary was interpolated based on 	 	 calculation of the slope(ie. rise over run) using the KNOWN elevations of 	 and the distance between the two macro-reach boundaries that exist 	 	 immediately upstream and downstream of this lower boundary.

RCH_LENGTH - The length of the macro-reach in metres.

RCH_MAGNIT - The Stream magnitude of the macro-reach. Stream Magnitude is derived from the BC Watershed Atlas.

RCH_ORDER - The Stream Order of the macro-reach. Stream Order is derived from the BC Watershed Atlas.

GRADIENT - The Gradient of the macro-reach as a percentage.

SINUOSITY - The Stream Channel length of the macro-reach in metres divided by the Stream Valley length of the macro-reach in metres. The sinuosity indicates the degree of wandering or winding of the macro-reach. A high sinuousity ( ie. > 1.5 ) indicates high channel movement, high sediment deposition and low stream energy.

THEME_ATTR - The ARC/INFO symbol number used to color theme the SIX standard macro-reach stream gradient classes.

Line Symbol          Line Color              Gradient Class

value

35                 Green                     0% - 2%

37                 Light Blue               >2% - 8%

38                 Majenta                  >8% - 12%

36                 Blue                     >12% - 16%

34                 Red                      >16% - 20%

41                 Mustard                  >20%

Note: 461 is the line symbol for Lake gradients only = 0%

WHEN_CREAT - Date when this macro-reach digital data record created

WS_CODE - The forty-five digit BC Watershed Atlas code that uniquely identifies the stream in which the macro-reach occurs.

GAZ_NAME - Gazetted Stream Name

ALIAS_1 - Alias Stream Name