ArcView Quality Assurance Tool

for

1999/2000 Fish Inventory Data

November 1999

Developed for the British Columbia Ministry of Fisheries

Fisheries Inventory Section

by

Geosense Consulting Ltd.

Nelson, BC

# Table of Contents

Page

New for 1999/2000 Fish QA Tool					iii

Introduction								1

Purpose							1

Limitations							1

Installation								2

## System Requirements						2

Required Files							2

Standalone ArcView Installation - Windows NT 4.0		2

Directory Structure of FishQA Files				3

GOAT ArcView Installation					4

ODBC Drivers							4

System Fonts							4

Loading the Extension - ArcView GOAT or 			5

Standalone ArcView

QA Procedure								7

Project Deliverables						7

## Running The QA Program –

###### Fish and Fish Habitat Map Features Table			7

Fish Data QA Tests							11

Fish and Fish Habitat Map Features Table			11

FDIS Reach Numbering Test					12

Testing Proximity to TRIM					12

Tools and Utilities							15

View Menus							15

Table Menus							17

# New for 1999/2000 Fish QA Tool

This is the second version of the ArcView Fish QA Tool.  The first version was developed for the 1998/1999 project year, and was primarily used for internal purposes by Ministry staff.  Much of the original core of the program remains,  however, there have been updates and enhancements made to the 1999/2000 version.  These are as follows:

## QA Processing

- The table testing has been updated to meet the  “Errata to Standards for Fish and Fish Habitat Map Features Table, Version 2.0, RIC 1998”.  Under the new standards there is only one data table submitted.  A Point table is no longer required.  The delivery format is simply one dBase table for the GIS features and one dBase metadata table.  The extension has been updated to handle the new table structure and formats.

- Error popup windows during the QA tests have been removed.  The QA report at the completion of the tests contains information regarding any errors that are detected. It will report on all tests whether an error is detected or not.

- The full watershed code is required under the “WS_CODE” column, a QA test checks that the formatting of the code is correct.

- All columns of the attribute table are examined during the feature code test to check that the fields requiring an entry for a specific feature code are filled.

- The proximity check has been revised so that is will exclusively check reaches, a selected set of features, or a selected set of reaches in a View.

## Tools and Utilities

- The tool that generates NID numbers can know handle newly added records. A user can add new records/features and then generate new NID values for the new entries.

- A table tool has been added to automatically generate “MAPNID” and “ILP_ID” field values

- A table tool has been added to automatically generate the abbreviated watershed code for the “AWSCODE” column from the full watershed code under the “WS_CODE” column

# Introduction

## Purpose

The Fish QA Tool is an ArcView extension that performs quality assurance tests on the GIS component of fish inventory data deliverables.  It is intended for use by Regional Fish Inventory specialists, their staff, and contractors compiling fish inventory projects. The QA extension performs a scan of the content and quality of contract deliverables and produces a report that lists detected problems.  In addition to the QA process, the extension also includes a number of tools to visualize and evaluate data relevant to fish inventory mapping and quality checking.  There are also a number of utilities that may be useful to a contractor during a mapping project.

The user of this extension will require a sound understanding of how ArcView displays and manages GIS data.  However, little or no outside support, such as from Arc/Info, should be required.

As an ArcView extension the set of tools can be loaded into existing ArcView projects. There are two modes of loading the extension. The first is a standalone mode that runs in native ArcView GIS.  The second mode of the tool loads into the ArcView BC Environment Geographic and Oracle Access Tool (GOAT).

## Limitations

The QA tool performs tests on the data that would be time consuming if done manually. The QA tool identifies potential problems in the spatial component of data in relation to the RIC standards manual  "Standards for Fish and Fish Habitat Maps, Version 2.0, RIC 1998" and "Errata to Standards for Fish and Fish Habitat Maps, RIC, April 1999",  Appendix 4 – The Fish and Fish Habitat Features Table.   These documents must be referred to during the use of the FishQA extension.  It should be recognized that the results of this QA process do not provide an overall pass or fail rating. It is possible that some errors detected by the QA procedures are not in fact errors in the data, but rather a reflection of project anomalies or of an authorized deviation from the standards.  The QA tests will identify discrepancies, but the final evaluation is left to the discretion of the Ministry and/or the contract monitor.

The QA procedure of the extension should be thought of as a checkpoint.  If notable structural or content problems are found in the data deliverables, the data may be returned to the contractor for repair.  If the data passes the QA tests and visual inspection, it may be accepted for further processing by the Ministry.

Installation

## System Requirements

The QA tool extension requires ArcView version 3.1 or later.  It will not function in earlier versions of ArcView.  The minimum hardware requirements for running ArcView 3.1 should suffice for operating the extension.

The QA extension was developed for the Window NT 4.0 operating system.  It has not been tested under any other Windows version. It may work in Windows95/98.  It will not work under Windows 3.1.

## Required Files

The QA tool refers to a number of files while performing tests on inventory data.  These files are provided with the QA tool extension and must be installed in order to use the extension properly. The files must be located in a directory called fishqa99, referenced by a path variable called USEREXT.   ArcView will refer to the location as %USEREXT%/fishqa99.  The USEREXT variable is used by ArcView to reference the location of extension files.  By default, ArcView looks for extension files in the /Ext32 or /Ext16 subdirectories under the ArcView program directory.  ArcView will also look for extension files located in the path stored in the USEREXT system variable.  The user must place the fishqa99.avx file and the fishqa99 directory of files into an appropriate location on the system.  For standalone users, the recommended installation is to create a directory on the C: drive called \userext and copy the fishqa99.avx file and the \fishqa99 directory structure into c:\userext.

### Standalone Arcview Installation - Windows NT 4.0

The FishQA tool is distributed in a compressed “.zip” file format.  The pathnames for the directory structure are stored within the compressed files.  To install, copy the fishqa99.zip file into an appropriate directory on the system, (e.g. c:\userext) .  Use PKUNZIP with the “-d” option to unzip the file and rebuild the directory structure.   If using WinZIP, you must check the “Use Folder Names” checkbox in the lower left corner of the “Extract” window to rebuild the directory structure.

The USEREXT system variable must be set to point to the directory of the FishQA extension file. To set a system variable in Windows NT 4.0, right click on the “My Computer” icon

on the desktop, select “Properties”, and then click the “Environment” tab.  This opens a window for setting system variables.  Under the “User Variables” set the variable USEREXT to the pathname where the fishqa99 files reside, ( e.g.  c:\userext)

Window NT 4.0 -  Setting system variable USEREXT:

NOTE:  Windows98 users must set the USEREXT variable in the AUTOEXEC.BAT file by adding the following line:   set USEREXT=c:\userext

Directory Structure of FishQA Files

The following is the directory of files delivered with the extension:

Fishqa99 Contents:

%USEREXT%\fshqa99.doc	· The ArcView Quality Assurance Tool for 1999/2000 Fish Inventory Data - User's Manual ( MS Word97 )

%USEREXT%\fishqa99.avx	· This is the ArcView extension file that contains the programs and menus that run the extension.

%USEREXT%\fishqa99\etc

\fonts - directory	· Contains windows font files that are needed for viewing BC Environment symbology. See below for installation.

· These fonts must be installed as Windows fonts.  To install the fonts, copy the *.ttf files and paste them into the “fonts” directory under your WINNT or WINDOWS installation directory.

\symbols - directory	· Contains symbol files as ArcView object dictionaries

%USEREXT%\fishqa99\template

· Contains INFO tables for feature codes and the dBase template for the fish features table

· These tables are  loaded by the extension and are prefixed with "FSH"

· Do not modify or delete these tables in the project, when the extension is unloaded, the tables are removed

· The required tables must be visible to ArcView when the QA extension is loaded or the extension will automatically unload.

### GOAT ArcView Installation

No installation is necessary for users of the MELP ArcView GOAT 5.x GIS system.  The FishQA extension can be loaded directly from a GOAT session.  File installation is handled by the BC Environment corporate data warehouse distribution.  Please contact the GIS coordinator in your office for more information.

ODBC Drivers

In order to connect to MS Access databases storing the FDIS tables, ArcView uses Windows ODBC drivers.  The QA extension will check for one of the following named connections:

MS Access 7.0 Database

MS Access 97 Database

MS Access Database

If none of the above connection names are found, the FDIS connection option will be disabled.  If you have ODBC drivers loaded for MS Access and the connection does not work,  check the ODBC setup on your PC.  You may need to rename the connection to one of the three listed names supported by the extension.

## Windows System Fonts

It is recommended that “small fonts” be used for the system display option.  Large fonts may cause some of the dialog window to not format properly.  The extension was developed with small fonts.  This is a limitation of ArcView software.

Loading the Extension - ArcView GOAT or Standalone ArcView

The extension installs menus in the View window called FishQA and FDIS, a menu in the Table window titled FishQA, and a tool button on the lower right row of buttons in the View window labelled with a “U”. The functions of these tools are described later in this document.

The extension operates under two modes, a standalone or GOAT mode. The functions and tools operate the same in each mode, the only difference is the View menus are loaded into the “goatView” window while running GOAT and load into the “View” window in standalone mode.  The extension automatically detects the mode when it installs.

The following steps through the menu choices for loading the Fish QA extension into a ArcView GOAT session.  The procedure is the same for standalone ArcView.

Step 1:  Start ArcView.

Step 2:  Activate “Project Manager” Window.

Step 3:  Select “Extensions”.

Step 4:  Click on the box beside the “1999 Fish Inventory QA Tool” and then Press OK.

Step 5:  Once the extension finishes loading, a dialog window opens to start a QA session.  See the section “Running The QA Program” below for a description of the QA process.

##### WARNING: DO NOT LOAD BOTH THE 1998 AND THE 1999 FISHQA EXTENSIONS AT THE SAME TIME!

QA Procedure

Project Deliverables

Before the QA tool is run, the deliverable files should be evaluated for naming convention and for completeness.  Refer to Section 5.0, the "Deliverable Checklist" in the "Errata to Standards for Fish and Fish Habitat Maps, RIC, April 1999",  Appendix 4 - The Fish and Fish Habitat Features Table,  for a detailed description of the delivery requirements.

1) Evaluate deliverables

The contractor is required to submit two data elements under the GIS deliverables. These are a Fish and Fish Habitat Map Features database file in dBase, and a Metadata database file in dBase.  Neither of these tables should be missing.  The features table may be generated manually or from the MS Access table generation tool from FDIS.  The FishQA tool does not differentiate either table creation processes.

e.g.	The deliverable files should appear similar as:

A4199809.dbf	- Fish and Fish Habitat Map Features database

M4199809.dbf	- Metadata table

NOTE:  Arc/Info files are not required and should not be submitted under the 1999 standards.

## Running The QA Program – Fish and Fish Habitat Map Features Table

The following section describes the steps to run the QA program. Where appropriate, there is reference made to sections in Appendix 4 of the "Errata to Standards for Fish and Fish Habitat Maps, RIC, April 1999".

Step 1:  Load FishQA Extension

Load the QA tool extension as described under the Installation section of this document. When the extension loads it will automatically open the QA Setup window.  If you are starting from this point you can skip to step 3, otherwise open the QA Setup window as described in step 2.

Step 2:  To start a QA session select “Start New QA Session”  from “FishQA” View menu.

Step 3:  QA Dialog Appears - Select Project Tables (Steps 3.1-3.3)

When the extension is finished loading a dialog window is opened.  From the dialog the user selects the metadata and features table for a single fish inventory project.  The user can optionally select the MS Access FDIS database for the project.  If an FDIS database is selected with the project the QA procedure will check for reach numbering and remain connected to the database so that the FDIS reaches and samples sites can subsequently be loaded as a theme in a View.

Step 3.1 Select Meta Table by pressing “open” button

## Meta Table

When the metadata table is selected the table is scanned for information about the project. It is important that the contractor submits a metadata table , and that the table conforms to the specification described in the standards manual, see Section 2.3 of the mapping standards manual.  If a metadata table is not submitted, the QA can proceed, however, the final report will not have any descriptive information about the project.

## Step 3.2 Select Fish Map Features Table by pressing “open” button   
  Features Table

The features file should be an dBase table with an .dbf file extension. Refer to Section 5.0 in the standards for a description of the naming conventions and format of the features table.

## Step 3.3 Select FDIS File by pressing “open” button

## FDIS Database

To connect to the FDIS “Reach_cards” and “S_site_card” tables the user must select the file named “fdisdat.mdb” associated with the project being tested.  A Features table must be selected before selecting an FDIS database.

NOTE: FDIS has its own built-in QA procedures

Step 4 Start QA Program

Once a features table is selected  the dialog “START” button is enabled. Press the start button to begin the QA tests.  If the cancel button is pressed, the choices are cleared and if FDIS was connected, the connection is closed.

Step 5 QA Report

At the completion of the QA program the results of the tests are summarized in a text window.  The report is also saved to a text file. The location of the file is shown at the bottom of the window.

The QA report lists the results of tests on the Feature table.

Each test will report on the number of errors found and then provide more specific information about the records affected.

When the QA program is completed the Feature table is added to the ArcView project. The table is prefixed with "QA" and has a suffix of the project number.  If errors are reported the user should open the tables and examine them for the reported errors. Use the field definition tool under the tables menu to check for column definition errors.  Attribute table errors will refer to the record number, MAPNID, watershed code, or ILP values when reporting errors related to a set of records.  It is up to the user to use the ArcView query tool or to browse the tables to examine these errors further.

Fish Data QA Tests

Fish and Fish Habitat Map Features Table

The QA procedures of this extension run checks against the Fish and Fish Habitat Map Features table described in Appendix 4,  "Errata to Standards for Fish and Fish Habitat Maps, RIC, April 1999".   The QA procedures directly follow the specifications for data structure and content for the features table, described under Section 4.2 of Appendix 4.

Please note, the extension does not support any output from the FDISMap program.

The following tests are run on the Features tables:

- Test if any fields are missing

- Test that the fields are in the correct order in the table

- Field definitions

Each field in the table is checked for its definition or placement in the table against the template table called “FSH.attrib_template”

- Test if there are any records with a blank MAPNID value

- Uniqueness of MAPNID values

Each record in the Features table refers to a unique feature in the project.  The MAPNID value is the identifier for each feature, therefore every record must have a unique MAPNID value.  Records without a MAPNID entry are reported.

IMPORTANT:  If the QA test reports non-unique or missing MAPNID values, this should be fixed before further QA continues.  Several of the QA reports use the MAPNID value to identify records with detected errors. If the values are not unique, the results of some tests will not be reliable for the non-unique MAPNID values.

- Non-standard feature codes

Each entry in the FCODE column is compared against the standard feature codes in the BC Environment lookup tables.

- Records missing FCODE

The QA program reports if there are any records that do not have a feature code entered.

- Check presence of Watershed Code and ILP

Each record should have an entry for watershed code or ILP number. The QA program reports records, by MAPNID,  that are missing an entry for both watershed code and ILP fields.  It is possible to have both a watershed code and an ILP number for a record.

- Check formatting of Watershed Code

Records with an entry in the “WS_CODE” columns are checked to make sure that the watershed code is formatted correctly.  It checks the length and spacing of the “-“ separator.

- Mandatory field entries for feature codes

This test will check that the feature code for each record has data entered in the fields that have a mandatory entry related to that feature type.  For example, a record containing a reach break feature code should have an entry under the “REACH_NO” field to indicate the reach number.  This test checks that data has been entered, however, the test does not evaluate the content of each field for the record.

In the final QA report record numbers and MAPNID’s are listed for records with mandatory data missing.  All columns are checked for each record, so it is possible for the same record to have multiple occurrences in the QA report.  The “FSH.Fcattributes” tables is used as the reference for this test.

FDIS Reach Numbering Test

When an FDIS Access database is selected in the QA setup window, a test is run on the “Reach_cards” table in the FDIS database to check the numbering sequence assigned to reach breaks on a stream.  This test is not done by the built-in QA routines of the FDIS MS Access application.  The QA program will connect to the MS Access database through the ODBC SQL drivers, retrieve the Reach_cards table and evaluate the numbering sequence for each unique instance of watershed code or ILP number.  It also checks decimal values in reach numbers.

There are no other tests run directly on the FDIS database at this time.

NOTE:  Reach break numbers are also stored in the Lake_Cards table.  The QA program will check this table for reach numbers that are missing from the Reach_Cards table for a given stream.  However, the Lake_Cards table does not store ILP values therefore it is possible that reach numbers reported missing on ILP streams are in fact reach breaks on lake features.

Testing Proximity to TRIM

The QA tool will check the spatial accuracy of point features in relation to a line theme.  This test is intended to check the placement of fish features on TRIM streams.  The QA Theme should be a point theme, and the reference theme should be TRIM water.  When the dialog opens it tries to find a line theme with the "twtr" string in its data source name.  This tool only checks the proximity of points to lines.  The standards specify that features should be within 10 meters of TRIM stream linework.  In reality, the points should be snapped exactly onto the TRIM water features.

NOTE: The map projection for the reference theme and the QA theme must be the same.

Step 1:  Load a point theme

The point theme to be tested can be generated from one of four sources:

- The features table – Use the tool “Convert UTM table to Albers Shapefile” described in the Tools section below.

- FDIS Reach_cards table – Follow step 1.1 below

- FDIS S_site_cards table – Follow step 1.1 below

- Add an Event Theme using UTM coordinates (use when the map base is UTM)

Step 1.1 Load Sample Sites and Reach Breaks from the view menu titled “FDIS”.

If you are not connected to a FDIS database, select the “Connect to FDIS” menu and select a MS Access database.

Step 2:  Load TRIM 1:20k water theme.

Use a TRIM water in the same map projection as the QA point theme

Step 3:  Select QA Proximity from the view menu titled “FishQA”

Step 4:  Select Proximal QA Theme.

If you use a point theme generated from the Features table as QA Theme, you have the option to only test the proximity of reach breaks to TRIM water.  Click the checkbox called “Check reaches only” if you are only interested at checking reach breaks.  The test can also work on a subset of the theme. If a selection exists and you only want to check the selected features, click the checkbox called “Use current selection”.  If both checkboxes are checked on, only reach breaks from the current selection are tested.

If the QA theme does not have an “FCODE” column in its database the “Check reaches only” checkbox will not be available.

Step 5:  Proximal Test identifies number of features outside of desired distance and   	highlights identified feature on map.

If features are detected outside the set tolerance, the number of features detected is displayed and the features become selected in the theme.  The features are highlighted on the display window using the current selection colour. . A button labelled “Show Table” will appear if features outside the tolerance are detected..  By clicking the button, the attribute table for the QA theme will open and promote the selected records to the top of the table.

Features, such as reach breaks outside of the 10 meter tolerance should be examined further, and may require repair.

NOTES:  - The results from the proximity test are not written to the QA Report.

- The line theme should have a spatial index on it. For large coverages it is advised that a spatial index be built prior to excuting the proximal test.  If the line theme is a layer from ArcStorm, the ArcStorm layer should be have a spatial index.

Tools and Utilities

The Fish QA extension includes several tasks and utilities that offer the user further QA ability and data processing.

## VIEW MENUS

"FishQA" Menu:

Start New QA Session

The QA dialog is started from the View menu titled “FishQA”  from the choice labelled “Start New QA Session”.

Convert UTM table to Albers Shapefile

This tool prompts the user to select a database table that has UTM co-ordinates in easting, northing, and zone fields and convert the locations to a new point shapefile in the Albers map projection.  This can be run on the attribute table for the dBase option or any other table that has UTM coordinates.  This tool does not handle Datum shifts from NAD27 to NAD83.

If the input table has a feature code field called FCODE, the program will attempt to assign symbols in a theme legend.

QA Proximity

Discussed in QA Procedure section above.

Build FCODE Legend

This tool will build a legend with BC Environment symbols and labels for a theme that has a feature code attribute.  The feature code must be stored in column name "fcode" for the program to work.

Open table to write UTM x,y

This tool captures an Albers co-ordinate from the display and writes the corresponding UTM and lat/long value to a table.  Below is a description how to use this tool:

- Ensure that the map projection of the base data themes is the BC Environment standards Albers Conic.  This tool does not work in any other projection.

- Initialize a table to receive the UTM values.  This can be either a new table, or an existing.   If existing,  the table must have been previously created with this tool.

- Once a table is open for writing its window will appear in the top left corner of the ArcView window.

- A tool button labelled "U" will become active automatically in the View GUI. When the user clicks on the map display ( Albers projection)  the UTM and lat/long in decimals degrees are written to a new record.

- Click on the “Stop writing to UTM table menu” and the tables is saved and closed.

- This tool is a modification of a GOAT tool called 'Report XY" which reports UTM and lat/long coordinates to the sceen.

Add XY, UTM lat/long to Albers Theme

This tool is similar to the Arc/Info function called ADDXY. It adds the UTM and lat/long co-ordinate to a point theme attribute table.  As an option it can add NID and MAP fields. If a 1:20,000 BCGS grid theme is loaded in the View, the user can optionally choose to have the mapsheet label written to the table. The user can then generate unique NID values for each mapsheet using a tool in the FishQA Table menu, described below.

IMPORTANT:  If points are edited or added to the theme, this tool must be re-run to update the co-ordinate values.

Snap Points to TRIM

This tool will snap point features from the active theme  to vertices of a line theme.  The active theme must be a point shapefile.  The snap theme must be a line theme. This tool is useful if locations are being entered for stream features that should be snapped to the stream linework.

"FDIS" Menu:

Connect/Disconnect from FDIS

This program prompts the user to select a FDIS MS Access file to connect with.  This requires the ODBC drivers for MS Access described under the Installation instructions.  Once connected, the menu choices listed below become active.

Load Sample Sites

When connected to an FDIS database, this tool will load the sample sites from the FDIS S_site_card table and convert the UTM locations to a shapefile in the Albers map projection.

Load Reach Breaks

When connected to an FDIS database, this tool will load the reach breaks from the FDIS reach_card table and convert the UTM locations to a shapefile in the Albers map projection.

## TABLES MENUS

"FishQA" Menu

Show Field definition

A dialog window allows a user to click on a field in a table and retrieve the field definition.  If the field selected matches the name of a field in the spatial or attribute standards, it reports whether definition matches the standard field definition.

The dialog window can be left open to check the definition of other fields. Simply select another field and press the "Def'n" button on the dialog.  This tool is useful to use after a QA report lists fields that do not match the standard format.

Generate NID Values

If a table has been modified by using the "Add XY" tool, described above,  it will have a field named "MAP" that stores the 1:20,000 mapsheet tag, and a field named "NID".  This tool will generate unique NID values for each record from each unique mapsheet.   If a “NID” field already exists in the table it will also be used.

The tool can be used to generate NID values for newly added records.  The program detects the highest NID value used for the mapsheet and starts the NID values for the new records at the next highest NID value.

This program will work on a selected set of records or the entire table.

NOTE: Once executed, the results cannot be undone.

Generate MAPNID/Generate ILP_ID

This tool will automatically generate the entries for the MAPNID and ILP_ID fields.  Both menus require that the NID and MAP or the ILPMAP and ILP fields to be present and entered correctly.  If values in any of these fields change, the MAPNID or ILP_ID values need to be re-generated with this tool, it is not automatic.

The program will not calculate the resultant field in the following cases:  ILP=0, ILPMAP or MAP = Blank, and NID=0

NOTE: Once executed, the results cannot be undone.

Generate AWSCODE

A new field for 1999 called “AWSCODE” contains the abbreviated watershed code.  This tool will create the abbreviated value, provided that the watershed code under the “WS_CODE” is formatted correctly.

NOTE: Once executed, the results cannot be undone.

Export UTM to FDIS Import

This dialog tool exports the MAP, NID, and UTM values from a table to a CSV (Comma Separate Value) file.  The output from this tool can be imported into an FDIS database using the "UTM Import" utility.  This tool does not check the content of the fields being exported.

Show Table Source FileName

This tool displays the pathname to the source data file for the active Table document.